Bending Setup

Aim:

- To make a bending setup using a strain gauge in a cantilever beam and measure the load using an amplifier and ADC converter.

Components Required:

- Cantilever Beam

- Support

- Strain Guage

- Amplifier (with ADC - Analog to Digital Converter)

Key formulas:

| Deflection | Strain | Stress |
| --- | --- | --- |
| δ=4PL3Ebh3= PL3EI | ε=6PLEbh2 | σ=6PLbh2 |

For detailed understanding visit: https://www.allaboutcircuits.com/textbook/direct-current/chpt-9/strain-gauges/

Types of Wheatstone Configurations

- Quarter Bridge Configuration

Fig (1) – Quarter Wheatstone bridge

- R1 = R2 = R3 = R4

Since All R are equal the bridge is balanced Vo = 0

Now due to strain R4 change,

R4 = R + ΔR

Using voltage divider principles:

Left midpoint voltage:

Vleft= Vcc × [ R3R1 + R3]

Right midpoint voltage:

Vright= Vcc ×[R + ΔR (R2 + (R + ΔR)]

Bridge output:

Vo = Vleft - Vright

Vo = Vcc × R2R + ΔR-12≈ (VccΔR4R)

- Half bridge Configuration

Fig (2) – Half Wheatstone Bridge

- R1 = R2 = R3 = R4

- If both the Strain Guage resistances vary in same direction they should be placed in same side of the bridge (Case 1)

- If both the Strain Guage resistances vary in opposite direction they should be placed in opposite side of the bridge (Case 2)

Case 1:
R3 = R + ΔR (Tension)
R4 = R − ΔR (Compression)

Vo=VccΔR2Rx

Case 2:

R1 = R + ΔR
R2 = R + ΔR

Vo=VccΔR2R

- Full Bridge configuration

Fig (3) – Full Wheatstone Bridge

R1 = R + ΔR (Tension)
R2 = R − ΔR (Compression)
R3 = R + ΔR (Tension)
R4 = R − ΔR (Compression)

Vo=VccΔRR

Gauge Factor (G.F) tells the change in electrical resistance due to the mechanical strain occurs.

G.F=ΔRR.ε

Amplifier (with ADC)